//  MRH Color Television/Fire Light with Control Pin

//   G. Bunza 2016

//

#define FireRED 12      // RED LED (Anode/Plus side) attaches to this pin

#define FireYEL 11      // YELLOW LED (Anode/Plus side) attaches to this pin

#define FireWHT 10      // WHITE LED (Anode/Plus side) attaches to this pin

#define change_delay  60

int control_pin = 14;   //Control Pin is Pin 14 (A0) Low/Ground is OFF

// OPEN (Unattachd) Pin is ON

void setup()

{

pinMode(FireRED, OUTPUT);

pinMode(FireYEL, OUTPUT);

pinMode(FireWHT, OUTPUT);

digitalWrite(FireRED, LOW);

digitalWrite(FireYEL, LOW);

digitalWrite(FireWHT, LOW);

pinMode (control_pin, INPUT_PULLUP);

}

void loop()

{

digitalWrite(FireRED, tvtuning(84));

digitalWrite(FireYEL, tvtuning(77));

digitalWrite(FireWHT, tvtuning(5));

delay (change_delay);

}

byte tvtuning (int time_on)  {

if (digitalRead(control_pin) == LOW)  return 0; //Eventually all OFF

if (random (0,100)< time_on) return 1;          // ON time_on% of the time

else return 0;

}