//   MRH Plays Pin Selected Sound Tracks with Control Pin

//   G. Bunza 2016                            Selected_Sound_Clips

//   Uses DFPlayer_Mini_Mp3 library provides functions for DFPlayer mini mp3 module

//

#include <SoftwareSerial.h>

#include <DFPlayer_Mini_Mp3.h>

SoftwareSerial mySerial(17, 16); // PRO MINI RX, PRO MINI TX

int busy_pin    =    15;

int control_pin =    14;       // Control Pin is Pin 14 (A0) Low/Ground is OFF

// OPEN (Unattachd) Pin is ON

#define sound_start_delay  0   //number of minutes to delay before starting play

#define num_clips          9  //number of sound tracks/clips on the Micro SD Memory Card

int del_tim = 4000;

int tctr, tctr2, i;

int num_select_pins = 11;    // number of clip selects

int select_pins[ ] = {2,3,4,5,6,7,8,9,10,11,12}; //sound clip select pins to select

//clips 0001.mp3,0002.mp3,...in order

//

void setup () {

pinMode (busy_pin, INPUT);

pinMode(control_pin,INPUT_PULLUP);

for (i=0; i< num_select_pins; i++) pinMode (select_pins[i],INPUT_PULLUP);

mySerial.begin (9600);

mp3_set_serial (mySerial);    //set softwareSerial for DFPlayer-mini mp3 module

mp3_reset ();

delay(100);

mp3_set_volume (4);

delay(100);

for (tctr2=0; tctr<sound_start_delay; tctr++) { //Setup delay: sound_start_delay minutes

for (tctr=0; tctr<7; tctr++) {

delay (10000);              //10 second delay X 6 = 1 minute total delay

}

}

}

void loop () {

if ((digitalRead(control_pin)==HIGH))

{              //check control pin to start

for (i=0; i< num_select_pins; i++)  {

if (digitalRead(select_pins[i])==LOW) mp3_play (i+1);  //  play clip i

while (((digitalRead(busy_pin))== LOW)&&((digitalRead(busy_pin))== LOW)) {tctr = 1; }  //wait for track to end

}

}

}