//  MRH Servo Slow Back and Forth with Control Pin

//   G. Bunza 2016

//

#include <SoftwareServo.h>

SoftwareServo servo1;

int i;

#define servo_delay 10     // incremental servo delay time in milliseconds (0.001 seconds)

#define servo_pause 1000  // incremental servo pause time before direction change (millisec)

#define servo_pin  3      //pin where servo is connected

#define servo_start 20    // Servo start position always less than servo_stop

#define servo_stop  168   // Servo start position

#define control_pin 14    //Control Pin is Pin 14 (A0) Low/Ground is OFF

// OPEN (Unattachd) Pin is ON

void setup()

{

servo1.attach(servo_pin);

servo1.write(servo_start);

pinMode (control_pin, INPUT_PULLUP);

}

void loop()

{

if (digitalRead(control_pin) == HIGH)  {

for (i = servo_start; i <= servo_stop; i++) {

servo1.write(i);

SoftwareServo::refresh();

delay(servo_delay);

}

delay(servo_pause);

for (i = servo_stop; i >= servo_start; i--) {

servo1.write(i);

SoftwareServo::refresh();

delay(servo_delay);

}

delay(servo_pause);

}

SoftwareServo::refresh();

}